Vizsgaremek

WAdmin kft.

Készítette:

Szalai Bálint

Tóth László

Lakatos Richárd

Tartalomjegyzék

Hálózati dokumentáció	3

Hálózati infrastruktúra	3

VLAN kialakítása	3

Második és harmadik rétegbeli redundáns megoldások	3

IPv4 és IPv6 címzési rendszer	3

Vezeték nélküli hálózat	3

Statikus és dinamikus forgalomirányítás	3

Statikus és dinamikus címfordítás	3

WAN-összeköttetéseket	3

Virtuális magánhálózat (VPN)	3

Programozott hálózatkonfiguráció	4

Forgalomirányítón megvalósított biztonsági funkciók, hardveres tűzfaleszköz	4

Linux és Windows kiszolgálók	4

Irodalomjegyzék	4

Mellékletek	4

Eszközök konfigurációja	4

Eszközlista:	4

Tesztelési Dokumentáció	5

Hálózat működésének tesztelése	5

Hálózati szolgáltatások tesztelése	5

Windows szolgáltatások tesztelése	5

Linux szolgáltatások tesztelése	5

# Hálózati dokumentáció

# Hálózati infrastruktúra

A hálózatban 3 router, 4 switch, 2 szerver, 1 wifi router, 3 vlan,  10 telephely található

# VLAN kialakítása

Az R2-ből ki jövő interface-t 3 Vlan interface-re bontottuk.

A Vlan1-be 3 switch található. A Vlan1 a 192.168.10.0-ás hálózat.
A Vlan2-be 30 állomás található. A Vlan2 a 192.168.20.0-ás hálózat
A Vlan3-ba 30 állomás található. A Vlan3 a 192.168.30.0-ás hálózat

A Vlan-okra arra a célra van szükség, hogy le limitáljuk a routerek használatát ezzel a megoldással költség hatékonyabb a hálózat felépítése.

# Második és harmadik rétegbeli redundáns megoldások

A második rétegbeli redundanciát az stp protokol biztosítja a R2 lan-ba, hogy ha megszakad 2 switch között a kapcsolat akkor ne áljon le az egész hálózat.

A harmadik rétegbeli redundanciát az R1-R2-R3 között az eigrp irányító protokol biztosítja.

# IPv4 és IPv6 címzési rendszer

A végberendezések IP címét DHCP és statikus módon osztottunk ki. Az Adminpc kivételével minden alhálózatban lévő végberendezések dinamikusan kapnak IP címeket.

A VLAN alhálózatokban az R2-őt használtuk DHCP szervernek.
Az R1 alhálózatában a HOME ROUTER-en kívül minden az R1-en keresztül kapja dinamikusan az IP címet.

A következő hálózattokat osztottuk ki DHCP segítségével:

192.168.1.0 (R1 (kivéve a Wifi router))

192.168.10.0| 192.168.20.0| 192.168.30.0 (R2 (kivéve a 3 Switch))

192.168.2.0 (R3 (kivéve a Server0))

R1

R2

R3

Sw1

Sw2

Sw3

ASA

Server0

Server1

Adminpc

Home router

Az R2-ben az ASA alhálózat-on kívül a Szerver0-át használjuk az IP címek dinamikus kiosztására.
Azok az eszközök amik statikus IP címmel rendelkeznek:

| Sorszám | Hálózatnév | szükséges Címek száma | Hálózatcím |
| --- | --- | --- | --- |
| 1. | R1-R2 | 2 | 172.16.1.0/30 |
| 2. | R1-R3 | 2 | 172.16.1.4/30 |
| 3. | R2-R3 | 2 | 172.16.1.8/30 |
| 4. | R1 Lan | 30 | 192.168.1.0/24 |
| 5. | Wifi R | 10 | 192.168.5.0/28 |
| 6. | R3 Lan1 | 10 | 192.168.2.0/24 |
| 7. | Asa szer. | 2 | 192.168.3.0/28 |
| 8. | R2 Vlan1 | 30 | 192.168.10.0/24 |
| 9. | R2 Vlan2 | 30 | 192.168.20.0/24 |
| 10. | R2 Vlan3 | 30 | 192.168.30.0/24 |
| 11. | Asa pc | 2 | 192.168.3.16/28 |
| 12. | Fa0/0 | 30 | 2001:db8:abcd::/56 |
| Eszköz név | Interface | Ip cím | Alhálózati maszk | Alapértelmezett átjáró |
| R1 | Se0/3/0 | 172.16.1.1 | 255.255.255.252 | -- |
| -- | Se0/3/1 | 172.16.1.5 | 255.255.255.252 | -- |
| -- | Fa0/1 | 192.168.1.1 | 255.255.255.0 | -- |
| R2 | Se0/1/0 | 172.16.1.2 | 255.255.255.252 | -- |
| -- | Se0/1/1 | 172.16.1.9 | 255.255.255.252 | -- |
| -- | G0/0/0 | -- | -- | -- |
| -- | G0/0/0.10 | 192.168.10.1 | 255.255.255.128 | 192.168.10.1 |
|  | G0/0/0.20 | 192.168.20.1 | 255.255.255.128 | 192.168.20.1 |
|  | G0/0/0.30 | 192.168.30.1 | 255.255.255.128 | 192.168.30.1 |
|  | G0/0/0.40 | 192.168.40.1 | 255.255.255.0 | 192.168.40.1 |
| Vlan1 | Valn1 | 192.168.10.2 | 255.255.255.0 | 192.168.10.1 |
| R3 | Se0/1/0 | 172.16.1.6 | 255.255.255.252 | -- |
| -- | Se0/1/1 | 172.16.1.10 | 255.255.255.252 | -- |
| -- | G0/0/2 | 192.168.100.1 | 255.255.255.240 | -- |
| -- | G0/0/0 | 192.168.2.1 | 255.255.255.0 | -- |
| Asa | G1/2 | 192.168.3.17 | 255.255.255.240 | -- |
| -- | G1/3 | 192.168.3.1 | 255.255.255.240 | -- |
| Server0 | Fa0 | 192.168.2.250 | 255.255.255.0 | 192.168.2.1 |
| Server1 | Fa0 | 192.168.3.250 | 255.255.255.240 | 192.168.3.1 |
| R1Sw1 | Vlan1 | 192.168.1.2 | 255.255.255.0 | 192.168.1.1 |
| R1Sw2 | Vlan1 | 192.168.1.4 | 255.255.255.0 | 192.168.1.1 |
| R3Sw1 | Vlan1 | 192.168.2.2 | 255.255.255.0 | 192.168.2.1 |
| Adminpc | Fa0 | 192.168.3.20 | 255.255.255.240 | 192.168.3.17 |
| WifiR. | I0/0 | 192.168.1.100 | 255.255.255.0 | 192.168.1.1 |
| -- | Wifi | 192.168.5.1 | 255.255.255.240 | -- |
| R1 | Fa0/0 | 2001:db8:abcd:0100::1 | /64 |  |
| publik ip bent |  |  |  |  |

# Vezeték nélküli hálózat

A vezeték nélküli alhálózatot az R1 Fa0/0 interfacén hoztuk létre, Home router segítségével alakítottuk át a vezetékes internet kapcsolatot vezeték nélküli wifi-re.

A hálózatba 10 eszköz csatlakozhat és a 192.168.5.0-ás hálózatból osztja az ip címeket.

ssid: WAdminWifi

Pass phrase: Pa$$w0rd

Titkosítás: WPA2-PSK

Típus: IEEE802.11N

# Statikus és dinamikus forgalomirányítás

Az EIGRP irányító protokollal érjük el hogy az alhálózatok közötti kommunikáció gyors és megbízható legyen.

# Statikus és dinamikus címfordítás

Dinamikus PAT:

192.168.0.0/16 hálózat számára R3 PAT végzi

# WAN-összeköttetéseket

A hálózatban serial (soros) kábeleket használtunk a routerek összekötésére hdlc protokol alkalmazásával.

# Virtuális magánhálózat (VPN)

A virtuális magánhálózat Dial-up VPN-nel lett konfigurálva az R3 routeren.

# Programozott hálózatkonfiguráció

# Forgalomirányítón megvalósított biztonsági funkciók, hardveres tűzfaleszköz

Az R1, R2 és R3 routereken privilegizált EXEC módot védő jelszó lett beállítva.

ssh:

ASA tűzfal:

Hálózatai:

| Hálózat | Szint |
| --- | --- |
| Gigabitethernet1/2: | 100 |
| Gigabitethernet1/3: | 50 |
| Gigabitethernet1/1: | 0 |

# Linux és Windows kiszolgálók

Windows szerver neve és ip címe:

A windows szerverre telepítettük az active directory domain services kiszolgálót. Létrehoztunk egy felhasználót, mely be tud lépni a tartományba. Alábbi képen látható:

# Szalai Bálint nevű felhasználót beléptettük az m013.local tartományba

Linux felhasználó neve:

# A linux szervert beléptettük az m013.local tartományba

# Irodalomjegyzék

A vizsgaremek készítéséhez segítségünkre voltak Fehér Ferenc tanár úr által közzétett segédletek.

# Mellékletek

# Eszközlista

# Eszközök konfigurációja

Eszköz neve:

Router1:

Current configuration : 1177 bytes

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

hostname Router

enable password Wadmin

ip dhcp pool R1

network 192.168.1.0 255.255.255.0

default-router 192.168.1.1

no ip cef

no ipv6 cef

license udi pid CISCO2811/K9 sn FTX101762P5-

spanning-tree mode pvst

interface FastEthernet0/0

no ip address

duplex auto

speed auto

interface FastEthernet0/1

ip address 192.168.1.1 255.255.255.0

duplex auto

speed auto

interface Serial0/3/0

ip address 172.16.1.1 255.255.255.252

clock rate 2000000

interface Serial0/3/1

ip address 172.16.1.5 255.255.255.252

clock rate 2000000

interface FastEthernet1/0

no ip address

duplex auto

speed auto

interface FastEthernet1/1

no ip address

duplex auto

speed auto

interface Vlan1

no ip address

shutdown

router eigrp 1

network 172.16.1.0 0.0.0.3

network 172.16.1.4 0.0.0.3

network 192.168.1.0

router rip

version 2

network 172.16.0.0

network 192.168.1.0

ip classless

ip flow-export version 9

line con 0

line aux 0

line vty 0 4

login

end

Router2:

Current configuration : 2167 bytes

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

hostname R2

enable password Wadmin

ip dhcp excluded-address 192.168.10.1 192.168.10.10

ip dhcp excluded-address 192.168.20.1

ip dhcp excluded-address 192.168.30.1

ip dhcp excluded-address 192.168.20.1 192.168.20.10

ip dhcp excluded-address 192.168.30.1 192.168.30.10

ip dhcp pool vlan2

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

ip dhcp pool vlan3

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

no ip cef

no ipv6 cef

username admin privilege 15 password 0 Wadmin

ip domain-name Wadmin.hu

spanning-tree mode pvst

interface Tunnel0

ip address 172.16.0.11 255.255.255.0

mtu 1476

tunnel source Serial0/1/0

tunnel destination 80.10.10.1

interface GigabitEthernet0/0/0

no ip address

duplex auto

speed auto

interface GigabitEthernet0/0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

interface GigabitEthernet0/0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

interface GigabitEthernet0/0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

interface GigabitEthernet0/0/0.40

encapsulation dot1Q 40

ip address 192.168.40.1 255.255.255.0

interface GigabitEthernet0/0/1

no ip address

duplex auto

speed auto

interface GigabitEthernet0/0/2

no ip address

duplex auto

speed auto

shutdown

interface Serial0/1/0

ip address 150.50.50.1 255.255.255.252

interface Serial0/1/1

ip address 172.16.1.9 255.255.255.252

interface Vlan1

no ip address

shutdown

interface Vlan2

mac-address 00e0.a36a.b601

no ip address

router eigrp 1

network 172.16.1.0 0.0.0.3

network 172.16.1.8 0.0.0.3

network 192.168.10.0 0.0.0.127

router rip

version 2

network 172.16.0.0

network 192.168.10.0

network 192.168.20.0

network 192.168.30.0

network 192.168.40.0

ip classless

ip flow-export version 9

access-list 10 permit 192.168.0.0 0.0.255.255

line con 0

line aux 0

line vty 0 4

access-class 10 in

login local

transport input ssh

end

Router3:

Current configuration : 2439 bytes

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

hostname R3

enable password Wadmin

aaa new-model

aaa authentication login felhasznalok local

aaa authorization network csoport local

no ip cef

no ipv6 cef

username admin privilege 15 password 0 Wadmin

username wadmin password 0 wadmin

username wadmin1 secret 5 $1$mERr$NU.I6./j.bHmf06vplFJt/

crypto isakmp policy 1

encr aes 256

authentication pre-share

group 5

crypto isakmp client configuration group VPNfelh

key csjelszo

pool IPCIMEK

netmask 255.255.255.0

crypto ipsec transform-set TSET esp-aes 256 esp-sha-hmac

crypto dynamic-map DMAP 1

set transform-set TSET

crypto map CMAP client authentication list felhasznalok

crypto map CMAP isakmp authorization list csoport

crypto map CMAP client configuration address respond

crypto map CMAP 1 ipsec-isakmp dynamic DMAP

ip domain-name Wadmin.hu

spanning-tree mode pvst

interface Tunnel0

ip address 10.255.255.1 255.255.255.0

mtu 1476

tunnel source Serial0/1/1

interface GigabitEthernet0/0/0

ip address 192.168.2.1 255.255.255.0

ip nat inside

duplex auto

speed auto

interface GigabitEthernet0/0/1

no ip address

duplex auto

speed auto

interface GigabitEthernet0/0/2

ip address 192.168.100.1 255.255.255.240

duplex auto

speed auto

interface Serial0/1/0

ip address 172.16.1.6 255.255.255.252

ip nat inside

interface Serial0/1/1

ip address 172.16.1.10 255.255.255.252

ip nat inside

clock rate 2000000

interface Serial0/2/0

ip address 192.60.0.1 255.255.255.0

ip nat outside

crypto map CMAP

interface Serial0/2/1

no ip address

clock rate 2000000

interface Vlan1

no ip address

shutdown

router eigrp 1

network 172.16.1.4 0.0.0.3

network 172.16.1.8 0.0.0.3

network 192.168.2.0

router ospf 1

log-adjacency-changes

ip local pool IPCIMEK 192.168.2.152 192.168.2.180

ip nat inside source list 1 interface Serial0/1/0 overload

ip nat inside source static tcp 192.168.2.1 80 192.60.0.3 80

ip classless

ip route 0.0.0.0 0.0.0.0 Serial0/2/0

ip flow-export version 9

access-list 10 permit 192.168.0.0 0.0.255.255

access-list 10 permit 192.60.0.0 0.0.0.255

access-list 10 permit 192.61.0.0 0.0.0.255

access-list 10 permit any

line con 0

line aux 0

line vty 0 4

access-class 10 in

transport input ssh

end

Switch0:

Current configuration : 1554 bytes

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

hostname Switch

spanning-tree mode pvst

spanning-tree extend system-id

interface FastEthernet0/1

switchport access vlan 30

switchport mode trunk

interface FastEthernet0/2

switchport access vlan 3

switchport mode access

interface FastEthernet0/3

switchport access vlan 20

switchport mode access

interface FastEthernet0/4

switchport access vlan 3

switchport mode trunk

interface FastEthernet0/5

switchport access vlan 30

switchport mode trunk

interface FastEthernet0/6

switchport access vlan 2

switchport mode access

interface FastEthernet0/7

interface FastEthernet0/8

interface FastEthernet0/9

interface FastEthernet0/10

interface FastEthernet0/11

interface FastEthernet0/12

interface FastEthernet0/13

interface FastEthernet0/14

interface FastEthernet0/15

interface FastEthernet0/16

interface FastEthernet0/17

interface FastEthernet0/18

interface FastEthernet0/19

interface FastEthernet0/20

interface FastEthernet0/21

interface FastEthernet0/22

interface FastEthernet0/23

interface FastEthernet0/24

interface GigabitEthernet0/1

interface GigabitEthernet0/2

interface Vlan1

no ip address

shutdown

interface Vlan10

ip address 192.168.10.2 255.255.255.0

interface Vlan20

ip address 192.168.20.2 255.255.255.0

interface Vlan30

ip address 192.168.30.2 255.255.255.0

line con 0

line vty 0 4

login

line vty 5 15

login

end

Switch1:

Current configuration : 1522 bytes

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

hostname Switch

spanning-tree mode pvst

spanning-tree extend system-id

interface FastEthernet0/1

switchport access vlan 30

switchport mode access

interface FastEthernet0/2

switchport access vlan 30

switchport mode trunk

interface FastEthernet0/3

switchport access vlan 20

switchport mode access

interface FastEthernet0/4

switchport access vlan 30

switchport mode trunk

interface FastEthernet0/5

interface FastEthernet0/6

interface FastEthernet0/7

interface FastEthernet0/8

interface FastEthernet0/9

interface FastEthernet0/10

interface FastEthernet0/11

interface FastEthernet0/12

interface FastEthernet0/13

interface FastEthernet0/14

interface FastEthernet0/15

interface FastEthernet0/16

interface FastEthernet0/17

interface FastEthernet0/18

interface FastEthernet0/19

interface FastEthernet0/20

interface FastEthernet0/21

interface FastEthernet0/22

interface FastEthernet0/23

interface FastEthernet0/24

interface GigabitEthernet0/1

interface GigabitEthernet0/2

interface Vlan1

no ip address

shutdown

interface Vlan2

no ip address

interface Vlan3

no ip address

interface Vlan10

ip address 192.168.10.4 255.255.255.0

interface Vlan20

ip address 192.168.20.4 255.255.255.0

interface Vlan30

ip address 192.168.30.4 255.255.255.0

line con 0

line vty 0 4

login

line vty 5 15

login

end

Switch2:

Current configuration : 1444 bytes

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

hostname Switch

spanning-tree mode pvst

spanning-tree extend system-id

interface FastEthernet0/1

switchport access vlan 30

switchport mode access

interface FastEthernet0/2

switchport access vlan 30

switchport mode trunk

interface FastEthernet0/3

switchport mode trunk

interface FastEthernet0/4

interface FastEthernet0/5

interface FastEthernet0/6

interface FastEthernet0/7

interface FastEthernet0/8

interface FastEthernet0/9

interface FastEthernet0/10

interface FastEthernet0/11

interface FastEthernet0/12

interface FastEthernet0/13

interface FastEthernet0/14

interface FastEthernet0/15

interface FastEthernet0/16

interface FastEthernet0/17

interface FastEthernet0/18

interface FastEthernet0/19

interface FastEthernet0/20

interface FastEthernet0/21

interface FastEthernet0/22

interface FastEthernet0/23

interface FastEthernet0/24

interface GigabitEthernet0/1

interface GigabitEthernet0/2

interface Vlan1

no ip address

shutdown

interface Vlan2

no ip address

interface Vlan3

no ip address

interface Vlan10

ip address 192.168.10.3 255.255.255.0

interface Vlan20

ip address 192.168.20.3 255.255.255.0

interface Vlan30

ip address 192.168.30.3 255.255.255.0

line con 0

line vty 0 4

login

line vty 5 15

login

end

Eszköz neve:

# Tesztelési Dokumentáció

# Hálózat működésének tesztelése

PC6 🡪 PC7

C:\>ping 192.168.1.3

Pinging 192.168.1.3 with 32 bytes of data:

Reply from 192.168.1.3: bytes=32 time<1ms TTL=128

Reply from 192.168.1.3: bytes=32 time<1ms TTL=128

Reply from 192.168.1.3: bytes=32 time=1ms TTL=128

Reply from 192.168.1.3: bytes=32 time<1ms TTL=128

Ping statistics for 192.168.1.3:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 0ms, Maximum = 1ms, Average = 0ms

PC6 🡪 PC12

C:\>ping 192.168.30.11

Pinging 192.168.30.11 with 32 bytes of data:

Request timed out.

Reply from 192.168.30.11: bytes=32 time=1ms TTL=126

Reply from 192.168.30.11: bytes=32 time=1ms TTL=126

Reply from 192.168.30.11: bytes=32 time=7ms TTL=126

Ping statistics for 192.168.30.11:

Packets: Sent = 4, Received = 3, Lost = 1 (25% loss),

Approximate round trip times in milli-seconds:

Minimum = 1ms, Maximum = 7ms, Average = 3ms

PC6 🡪PC1

C:\>ping 192.168.20.11

Pinging 192.168.20.11 with 32 bytes of data:

Request timed out.

Reply from 192.168.20.11: bytes=32 time=6ms TTL=126

Reply from 192.168.20.11: bytes=32 time=6ms TTL=126

Reply from 192.168.20.11: bytes=32 time=6ms TTL=126

Ping statistics for 192.168.20.11:

Packets: Sent = 4, Received = 3, Lost = 1 (25% loss),

Approximate round trip times in milli-seconds:

Minimum = 6ms, Maximum = 6ms, Average = 6ms

PC6 →PC0

C:\>ping 192.61.0.3

Pinging 192.61.0.3 with 32 bytes of data:

Reply from 192.61.0.3: bytes=32 time<1ms TTL=128

Reply from 192.61.0.3: bytes=32 time=3ms TTL=128

Reply from 192.61.0.3: bytes=32 time<1ms TTL=128

Reply from 192.61.0.3: bytes=32 time<1ms TTL=128

Ping statistics for 192.61.0.3:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 0ms, Maximum = 3ms, Average = 0ms

PC6 →PC15

C:\>ping 192.168.2.2

Pinging 192.168.2.2 with 32 bytes of data:

Reply from 192.168.2.2: bytes=32 time=5ms TTL=128

Reply from 192.168.2.2: bytes=32 time<1ms TTL=128

Reply from 192.168.2.2: bytes=32 time<1ms TTL=128

Reply from 192.168.2.2: bytes=32 time<1ms TTL=128

Ping statistics for 192.168.2.2:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 0ms, Maximum = 5ms, Average = 1ms

# Hálózati szolgáltatások tesztelése

Switch0:

1 default active Fa0/7, Fa0/8, Fa0/9, Fa0/10

Fa0/11, Fa0/12, Fa0/13, Fa0/14

Fa0/15, Fa0/16, Fa0/17, Fa0/18

Fa0/19, Fa0/20, Fa0/21, Fa0/22

Fa0/23, Fa0/24, Gig0/1, Gig0/2

2 VLAN0002 active Fa0/6

3 VLAN0003 active Fa0/2

4 VLAN0004 active

10 VLAN0010 active

20 VLAN0020 active Fa0/3

30 VLAN0030 active

1002 fddi-default active

1003 token-ring-default active

1004 fddinet-default active

1005 trnet-default active

VLAN Type SAID MTU Parent RingNo BridgeNo Stp BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

1 enet 100001 1500 - - - - - 0 0

2 enet 100002 1500 - - - - - 0 0

3 enet 100003 1500 - - - - - 0 0

4 enet 100004 1500 - - - - - 0 0

10 enet 100010 1500 - - - - - 0 0

20 enet 100020 1500 - - - - - 0 0

30 enet 100030 1500 - - - - - 0 0

1002 fddi 101002 1500 - - - - - 0 0

1003 tr 101003 1500 - - - - - 0 0

1004 fdnet 101004 1500 - - - ieee - 0 0

1005 trnet 101005 1500 - - - ibm - 0 0

VLAN Type SAID MTU Parent RingNo BridgeNo Stp BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

Remote SPAN VLANs

------------------------------------------------------------------------------

Primary Secondary Type Ports

------- --------- ----------------- ------------------------------------------

Switch# show interface trunk

Port Mode Encapsulation Status Native vlan

Fa0/1 on 802.1q trunking 1

Fa0/4 on 802.1q trunking 1

Fa0/5 on 802.1q trunking 1

Port Vlans allowed on trunk

Fa0/1 1-1005

Fa0/4 1-1005

Fa0/5 1-1005

Port Vlans allowed and active in management domain

Fa0/1 1,2,3,4,10,20,30

Fa0/4 1,2,3,4,10,20,30

Fa0/5 1,2,3,4,10,20,30

Port Vlans in spanning tree forwarding state and not pruned

Fa0/1 1,2,3,4,10,20,30

Fa0/4 1,2,3,4,10,20,30

Fa0/5 1,2,3,4,10,20,30

Switch1:

Switch#show vlan

VLAN Name Status Ports

---- -------------------------------- --------- -------------------------------

1 default active Fa0/5, Fa0/6, Fa0/7, Fa0/8

Fa0/9, Fa0/10, Fa0/11, Fa0/12

Fa0/13, Fa0/14, Fa0/15, Fa0/16

Fa0/17, Fa0/18, Fa0/19, Fa0/20

Fa0/21, Fa0/22, Fa0/23, Fa0/24

Gig0/1, Gig0/2

2 VLAN0002 active

3 VLAN0003 active

4 VLAN0004 active

10 VLAN0010 active

20 VLAN0020 active Fa0/3

30 VLAN0030 active Fa0/1

1002 fddi-default active

1003 token-ring-default active

1004 fddinet-default active

1005 trnet-default active

VLAN Type SAID MTU Parent RingNo BridgeNo Stp BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

1 enet 100001 1500 - - - - - 0 0

2 enet 100002 1500 - - - - - 0 0

3 enet 100003 1500 - - - - - 0 0

4 enet 100004 1500 - - - - - 0 0

10 enet 100010 1500 - - - - - 0 0

20 enet 100020 1500 - - - - - 0 0

30 enet 100030 1500 - - - - - 0 0

1002 fddi 101002 1500 - - - - - 0 0

1003 tr 101003 1500 - - - - - 0 0

1004 fdnet 101004 1500 - - - ieee - 0 0

1005 trnet 101005 1500 - - - ibm - 0 0

VLAN Type SAID MTU Parent RingNo BridgeNo Stp BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

Remote SPAN VLANs

------------------------------------------------------------------------------

Primary Secondary Type Ports

------- --------- ----------------- ------------------------------------------

Switch#show interface trunk

Port Mode Encapsulation Status Native vlan

Fa0/2 on 802.1q trunking 1

Fa0/4 on 802.1q trunking 1

Port Vlans allowed on trunk

Fa0/2 1-1005

Fa0/4 1-1005

Port Vlans allowed and active in management domain

Fa0/2 1,2,3,4,10,20,30

Fa0/4 1,2,3,4,10,20,30

Port Vlans in spanning tree forwarding state and not pruned

Fa0/2 1,2,3,4,10,20,30

Fa0/4 1,2,3,4,10,20,30

Switch2:

Switch#show vlan

VLAN Name Status Ports

---- -------------------------------- --------- -------------------------------

1 default active Fa0/4, Fa0/5, Fa0/6, Fa0/7

Fa0/8, Fa0/9, Fa0/10, Fa0/11

Fa0/12, Fa0/13, Fa0/14, Fa0/15

Fa0/16, Fa0/17, Fa0/18, Fa0/19

Fa0/20, Fa0/21, Fa0/22, Fa0/23

Fa0/24, Gig0/1, Gig0/2

2 VLAN0002 active

3 VLAN0003 active

4 VLAN0004 active

10 VLAN0010 active

20 VLAN0020 active

30 VLAN0030 active Fa0/1

40 VLAN0040 active

1002 fddi-default active

1003 token-ring-default active

1004 fddinet-default active

1005 trnet-default active

VLAN Type SAID MTU Parent RingNo BridgeNo Stp BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

1 enet 100001 1500 - - - - - 0 0

2 enet 100002 1500 - - - - - 0 0

3 enet 100003 1500 - - - - - 0 0

4 enet 100004 1500 - - - - - 0 0

10 enet 100010 1500 - - - - - 0 0

20 enet 100020 1500 - - - - - 0 0

30 enet 100030 1500 - - - - - 0 0

40 enet 100040 1500 - - - - - 0 0

1002 fddi 101002 1500 - - - - - 0 0

1003 tr 101003 1500 - - - - - 0 0

1004 fdnet 101004 1500 - - - ieee - 0 0

1005 trnet 101005 1500 - - - ibm - 0 0

VLAN Type SAID MTU Parent RingNo BridgeNo Stp BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

Remote SPAN VLANs

------------------------------------------------------------------------------

Primary Secondary Type Ports

------- --------- ----------------- ------------------------------------------

Switch#show interface trunk

Port Mode Encapsulation Status Native vlan

Fa0/2 on 802.1q trunking 1

Fa0/3 on 802.1q trunking 1

Port Vlans allowed on trunk

Fa0/2 1-1005

Fa0/3 1-1005

Port Vlans allowed and active in management domain

Fa0/2 1,2,3,4,10,20,30,40

Fa0/3 1,2,3,4,10,20,30,40

Port Vlans in spanning tree forwarding state and not pruned

Fa0/2 1,2,3,4,10,20,30,40

Fa0/3 40

# Windows szolgáltatások tesztelése

# Linux szolgáltatások tesztelése